Metal ball valve (Q41/341/641/941F/H/Y)

Introduce:

Ball valve , opening and closing parts (ball) driven by the valve stem, and around the axis of the ball valve for rotating movement of the valve. It can also be used for fluid regulation and control, the hard seal V-type ball valve with V-type core and surfacing carbide metal valve seat has a strong shear force, especially suitable for containing fiber, small solid materials and other media. The multi-pass ball valve in the pipeline can not only flexibly control the medium confluence, shunt, and flow switch, but also can close any channel and make the other two channels connected. This type of valve shall generally be installed horizontally in the pipeline. Ball valve according to the driving mode is divided into: pneumatic ball valve, electric ball valve, manual ball valve.

Main parts and materials

| Name/Material | WCB | CF8 | CF3 | CF8M | CF3M |
| --- | --- | --- | --- | --- | --- |
| Body | WCB | CF8 | CF3 | CF8M | CF3M |
| Ball/Stem | WCB | CF8 | CF3 | CF8M | CF3M |
| Seat | PTFE / RPTFE/PPL |  |  |  |  |
| Bolt/Nut | B7/2H | B8/8 | B8/8 | B8M/8M | B8M/8M |

Technical Specification

| Design&Manufacture Standard | GB/T12237 | API 6D |
| --- | --- | --- |
| FTF Dimension Standard | GB/T12221 | ASME B16.10 |
| Flange standard | HG/T20592 | ASME B16.5 |
| Inspection & Test Standard | HG/T13927 | API 598 |

The main connection size

| DN (mm | L | D | D1 | D2 | b | Z-Φd | H | W |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q41F-16C Q41F-16P Q641F-16C Q641F-16P BQ41F-16C BQ941F-16P |  |  |  |  |  |  |  |  |
| 15 | 130 | 95 | 65 | 45 | 14 | 4φ14 | 78 | 130 |
| 20 | 130/140 | 105 | 75 | 55 | 14 | 4φ14 | 84 | 130 |
| 25 | 140/150 | 115 | 85 | 65 | 14 | 4φ14 | 95 | 160 |
| 32 | 165 | 135/140 | 100 | 78 | 16 | 4φ18 | 150 | 160 |
| 40 | 165/180 | 145/150 | 110 | 85 | 16 | 4φ18 | 150 | 180 |
| 50 | 203/200 | 160/165 | 125 | 100 | 16 | 4φ18 | 170 | 200 |
| 65 | 222/220 | 180/185 | 145 | 120 | 18 | 4φ18 | 195 | 240 |
| 80 | 241/250 | 195/200 | 160 | 135 | 20 | 8φ18 | 215 | 240 |
| 100 | 305/280 | 215/220 | 180 | 155 | 20 | 8φ18 | 250 | 280 |
| 125 | 356/320 | 245/250 | 210 | 185 | 22 | 8φ18 | 265 | 300 |
| 150 | 394/360 | 280/285 | 240 | 210 | 24 | 8φ23 | 270 | 300 |
| 200 | 400/457 | 335/340 | 295 | 265 | 26 | 12φ23 | 330 | 350 |
| 250 | 533 | 405 | 355 | 320 | 30 | 12φ26 | 450 | 400 |
| 300 | 610 | 460 | 410 | 375 | 30 | 12φ26 | 540 | 500 |